Stewart 51 Partner , LLC

SUGGESTED INSPECTION REPORT

for the

Stewart S51 Mustang

Revision C

August 5, 2012

Check www.stewart51.info for current edition.

NOTE:  It is the owner’s responsibility to ensure that his or her Stewart 51 Mustang is properly inspected and maintained at intervals required by regulation and good operating practice.  Each Stewart 51 Mustang is unique with its own equipment, accessories, and engine.  This suggested checklist is a guideline and must be changed, adjusted, and added to as each individual aircraft may require due to differences in installed equipment, accessories, and powerplants.

Aircraft Registration Number:   _____________   Aircraft Serial Number:  _____________   Aircraft Total Time:  _____________

Engine Model:   _________________________   Engine Serial Number: _______________   Engine Total Time:  _____________

Propeller Model Number:   ___________________________    Propeller Serial Number:  _________________________________

Initial on blank under 50, 100, 500, or 1,000 Hour interval.

In addition to the interval initialed below, this Inspection will also be performed as an Annual Inspection. (Initial if applicable):     ___

Date Inspection Begun:  _______________________

Where the terms “Inspect,” or  “Check” are used in this suggested report, those terms describe visual inspection or checks of items, and do not insinuate or specify dye penetrant, magnaflux, X-Ray, or other means of non-destructive processes.  “Inspect” and “Check” for purposes of this Suggested Inspection Report mean human visual observation.

A.  Operational Inspection.				50	100	500	1000

1.   Check aircraft condition for ground run of engine.	 	___	___	 ___	   ___

2.   Check fuel pump and fuel tank selector settings.		___	___	 ___	   ___

3.   Check all gages and instruments for proper

indications with master off, then with master on.    	___	___	 ___	   ___

4.   Start engine.  Check all gages and instruments

for proper operation and indications at idle.	 	___	 ___	 ___	   ___

5.   Check alternator performance and back up

electrical system as installed.			 	___	 ___	 ___	   ___

6.   After warm up, hold brakes and advance throttle

to 2500 engine RPM.  Check oil pressure, coolant

pressure for normal operating ranges, all gages and

instruments for proper operation and indications.	 	___	  ___	   ___	   ___

7.   Check ignition systems independently as installed. 	___	  ___	   ___	   ___

8.   Check propeller cycling and manifold pressure

indications.					 	___	  ___	   ___	   ___

9.   Retard throttle.  Check engine idle.  Shut down.	 	___	  ___	   ___	   ___

10. Check all gages and instruments for proper

indications.					 	___	  ___	   ___	   ___

B.  Engine Group						50	100	500	1000

1.  Remove engine cowling.			 	___	   ___	   ___	    ___

2.  Inspect engine for oil, fuel, and coolant leaks.	 	___	   ___	   ___	    ___

3.  Remove one set of spark plugs and perform

compression check while engine is warm.		 		   ___	   ___	    ___

4.  Clean and inspect cowling for cracks, distortion,

and loose or missing fasteners.			 	___	   ___	   ___	    ___

5.  Drain oil sump while engine is still warm.	 	___	   ___	   ___	    ___

6.  Remove full flow oil filter, open, and inspect for

metal and foreign material.			 	___	   ___	   ___	    ___

7.  Install new oil filter and safety as necessary.	 	___	   ___	   ___	    ___

50	100	500	1000

8.  Inspect oil temperature sending unit for leaks,

security, and wiring.					 ___	   ___	   ___	    ___

9.   Inspect oil lines, hoses, and fittings for leaks,

security, chafing, dents, and cracks.			 ___	   ___	   ___	    ___

10. Clean and inspect oil heat exchanger or oil

radiator as installed.  Verify that there is no

damage to fins or radiator from foreign objects.	 		   ___	   ___	    ___

11. Add proper quantity and grade oil.		 	 ___	   ___	   ___	    ___

12. Inspect engine mount for condition, cracks, and

damage.					 	 ___	   ___	   ___	    ___

13. Disconnect and inspect all ignition leads for

chafing, condition.					 ___	   ___	   ___	    ___

14. Remove all spark plugs.  Check condition

and clean.  Adjust gaps as necessary.		 	 ___	   ___	   ___	    ___

15. Check  condition of distributors, electronic

ignition components, magneto components

as installed.					 	 ___	   ___	   ___	    ___

16. Check timing components of distributors,

magnetos, internally and timing to engine.			   ___	   ___	    ___

17. Remove induction air filter, if installed, and clean

per manufacturers instructions or replace.	 	 ___	   ___	   ___	    ___

18. Remove and clean injector nozzles.				   ___	   ___	    ___

19. Inspect induction airbox for loose hardware,

cracks, and proper operation.				 ___	   ___	   ___	    ___

20. Inspect fuel injection hardware for rigging

chafing lines, security of linkage attach points,

and proper operation.					 ___	   ___	   ___	    ___

21. Inspect intake manifold for cracks, proper hose

clamps, and attachments.			 	 ___	   ___	   ___	    ___

22. Inspect all air inlet ducts and hoses for holes,

chafing, condition and secure clamp attachment .	 	 ___	   ___	   ___	    ___

23. Inspect all flexible fuel lines.  Replace at 5 years.	 	 ___	   ___	   ___	    ___

24. Clean gascolator bowl and screen as installed.			   ___	   ___	    ___

25. Inspect fuel system for leaks.			 	 ___	   ___	   ___	    ___

26. Inspect condition and operation of engine driven

and electric fuel pumps.  Clean or change

filter elements as required.				 ___	   ___	   ___	    ___

27. Overhaul or replace fuel pumps at 5 years or

manufacturer’s recommendation. 			 	   ___	   ___	    ___

28. Inspect hoses, wires, clamps, condition, and

operation of vacuum pumps as installed.			 ___ 	   ___	   ___	    ___

29. Replace vacuum pumps as recommended

by manufacturer.					 	   ___	   ___	    ___

30. Inspect throttle, mixture, alternate air, and

propeller controls for security, proper safety

wiring, cotter pin installation, and other

safety methods.  Check for proper operation,

range of travel, chafing, and condition.			 ___ 	   ___	   ___	    ___

31. Inspect exhaust stacks for cracks, missing

fasteners, condition of gasket.  Rotate stack

assemblies front to rear and vice versa as desired.   	 ___	   ___	   ___	    ___

32. Inspect breather tubes for obstructions, condition,

chafing, blockage, and clamp security.		 	 ___	   ___	   ___	    ___

33. Remove valve covers.  Inspect heads for cracks,

coolant leaks, and foreign material.			 ___	   ___	   ___	    ___

34. Inspect  valve rocker assembly and components

for cracks, wear, security of fasteners, and condition.	 ___	   ___	   ___	    ___

35. Check condition of valves, valve springs, seals,

keepers, and guides.					 ___	   ___	   ___	    ___

50	100	500	1000

36. (Solid Lifters) Check clearance between valve

rocker and valve stem per valve/rocker

manufacturer’s instructions.				 ___	   ___	   ___	    ___

37. Inspect valve cover spray bars as installed for

security, blockage, and proper operation.			 ___	   ___	   ___	    ___

38. Inspect engine block for cracks, coolant leaks, oil

leaks, and security of fasteners.				 ___	   ___	   ___	    ___

39. Inspect machined engine mounts for cracks,

security of fasteners, and condition.			 ___	   ___	   ___	    ___

40. Inspect rubber engine mount bushings.  Replace as

required or at engine overhaul.					   ___	   ___	    ___

41. Inspect firewall seals.					 ___	   ___	   ___	    ___

42. Inspect condition and security and proper routing of

drive mechanism, wiring, mounting hardware and

brackets, cooling hoses and ducts, and proper

installation of :	Alternator			 ___	   ___	   ___	    ___

Starter				 ___	   ___	   ___	    ___

43. Inspect supercharger or turbocharger as installed.

Inspect for heat damage, condition and security of

clamps, fasteners.  Remove thermal insulation,

check turbine wheel for motion, damaged blades.

check for air, exhaust, and oil leaks.			  ___	   ___	   ___	    ___

44. Overhaul engine at 500 hours operation or on condition.		   ___	   ___	    ___

C. Propeller Group					50	100	500	1000

1. Inspect spinner and back plate for cracks.		 	___ 	   ___	   ___	    ___

2.  Inspect blades for nicks and cracks.		 	___   	   ___	   ___	    ___

3.  Inspect for grease and oil leaks.			 	___	   ___	   ___	    ___

4.  Lubricate propeller per manufacturer lubrication chart.	 	   ___	   ___	    ___

5.  Inspect complete spinner and spinner

mounting bulkheads for security, chafing,

cracks, deterioration, wear, and correct

installation.						 ___	   ___	   ___	    ___

6.  Inspect propeller mounting bolts and safeties.		 ___	   ___	   ___	    ___

7.  Inspect hub parts for cracks and corrosion.			   ___	   ___	    ___

8.  Rotate blades and check for tightness in hub.		 ___	   ___	   ___	    ___

9.  Remove propeller and clean sludge from

propeller and shaft.					 	   ___	   ___	    ___

10. Inspect complete propeller assembly for security,

chafing, deterioration, wear, and correct

installation.						 ___ 	   ___	   ___	    ___

11. Overhaul propeller at manufacturer’s

recommended intervals.				 	   ___	   ___	    ___

12. Inspect propeller governor for condition,

security, leaks, condition of drive mechanism.		 ___	   ___	   ___	    ___

13. Overhaul or replace governor per manufacturer’s

recommendation.						   ___	   ___	    ___

D.  Propeller Speed Reduction Unit				50	100	500	1000

1.  Check housing for cracks and leaks.		 	___	   ___	   ___	    ___

2.  Inspect all seams and joints for leaks, proper sealing.	___	   ___	   ___	    ___

3.  Inspect all hoses and fittings for condition, chafing, and

leaks.  Check connections for tightness and proper safeties. ___	   ___	   ___	    ___

4.  Check gear backlash (per Dave Bogue in Rivethead).		   ___	   ___	    ___

5.  Check PSRU oil supply tank as installed for proper quantity,

condition.  Inspect filler cap seal and replace as necessary.	___  	   ___	   ___	    ___

6.  Remove and inspect PSRU oil filter as installed for metal.		   ___	   ___	    ___

7.  Inspect chip detector as installed for metal.  Inspect

wiring and connections for security, proper safety.		___  	   ___	   ___	    ___

8.  Remove PSRU.  Disassemble and inspect all bearings,

gears for condition and wear.  Replace parts as necessary.

Reassemble using new seals, O Rings, and gaskets. For

PSRU with lubrication system independent of engine

lubrication system,  refill with new oil of proper

specifications to proper quantity.			   	   	   ___	    ___

E.  Cockpit Group					50	100	500	1000

1.  Inspect canopy plexiglass for cracks, crazing, clarity.		   ___	   ___	    ___

2.  Inspect canopy frame for cracks, missing fasteners,

security of fasteners.						   ___	   ___	    ___

3.  Check canopy cranking mechanism for security, operation,

and condition.  Lubricate as necessary.				   ___	   ___	    ___

4.  Inspect Canopy Emergency Release mechanism for

condition, security, and proper operation.  Re-safety.		   ___	   ___	    ___

5.  Inspect windshield center glass and side glasses for cracks,

crazing, and clarity.						   ___	   ___	    ___

6.  Inspect windshield structure for loose or missing fasteners.

Remove inspection panels in front of windshield frame.		   ___	   ___	    ___

7.  Inspect seat upholstery for condition, tears, security.		   ___	   ___	    ___

8.  Inspect harnesses for condition, tears, mounting hardware

and proper operation.						   ___	   ___	    ___

9.   Inspect trim controls and operation.				   ___	   ___	    ___

10. Inspect rudder pedals for condition, security, hardware,

proper connections and travel.					   ___	   ___	    ___

11. Inspect brake master cylinders for condition, leaks,

security, and proper operation.					   ___	   ___	    ___

12. Inspect rudder cables and rods in cockpit area for proper

attachment, routing, chafing, safeties, and condition.	 	   ___	   ___	    ___

13. Remove seat pans, cockpit flooring.  Inspect all wiring

in cockpit for condition, security, chafing, proper routing.		   ___	   ___	    ___

14. Inspect all ducts and hoses in cockpit area for condition,

security, chafing, and proper routing.				   ___	   ___	    ___

15. Inspect all hydraulic lines and connections for condition,

security, chafing, routing, and leaks.				   ___	   ___	    ___

16. Remove access panels in wing center section.  Inspect

fuel selector valve, all fuel lines in center wing section

for security, condition, chafing, routing, and leaks.		   ___	   ___	    ___

17. Replace all flexible fuel lines each five years, or Teflon

hoses on condition.						   ___	   ___	    ___

50	100	500	1000

18. Inspect control sticks for condition, security, proper

travel.  Check all hardware and safeties.  Lubricate

pivots, joints, and bearings.					   ___	   ___	    ___

19. Inspect torque tube for condition, chafing, proper travel.

Lubricate.							   ___	   ___	    ___

20. Inspect aileron bell crank at rear spar for condition,

proper travel, excessive play or wear, security of hardware,

proper safeties.  Inspect rod ends for condition, security,

excessive play or wear, security of jam nuts and

attach bolts.  Lubricate.				 	   ___	   ___	    ___

21. Inspect flap actuator bracket for condition, cracks,

proper hardware, and security.					   ___	   ___	    ___

22. Inspect flap motor/actuator and hardware for condition,

security, proper wiring	.  Lubricate.		.		   ___	   ___	    ___

23. Inspect landing gear power pack as installed for condition,

proper mounting, wiring, fluid line connections, and leaks.		   ___	   ___	    ___

24. Service fluid reservoir as necessary with proper fluid to

proper level as installed.					   ___	   ___	    ___

25. Inspect aircraft battery(ies) and connections as installed

for condition, security, chafing, and proper installation.

Check condition and operation of vents and drains.

Inspect battery area / container(s) for corrosion.  Clean

and treat as necessary.						   ___	   ___	    ___

26. Inspect back side of all instruments, switches, equipment,

avionics installations for condition, proper wiring, security,

hoses and lines for routing, chafing, attachment,

and condition.						   ___	   ___	    ___

27. Inspect and test all circuit breakers, switches, equipment,

in cockpit and on panel for proper wiring, hoses, and

ducts for proper installation and operation.			   ___	   ___	    ___

28. Check all installed lights (navigation, strobe, landing,

cabin) for operation.						   ___	   ___	    ___

29. Inspect all vacuum and instrument air filters as installed.

Replace as necessary.						   ___	   ___	    ___

30. Check altimeter for proper operation and reading	.		   ___	   ___	    ___

31. Inspect fuel selector valve for condition and operation.

Inspect valve, lines, and connections for leaks.  Lubricate.		   ___	   ___	    ___

32. Inspect condition and operation of cabin ventilation outlets,

controls, and ducts.						   ___	   ___	    ___

33. Inspect all four wing attach brackets for damage,

security, loose and missing rivets and bolts.			   ___	   ___	    ___

34. Check wing attach through bolts for condition, security.		   ___	   ___	    ___

F.  Fuselage					 	50	100	500	1000

1.  Remove inspection panels, plates, and all fairings.			   ___	   ___	    ___

2.  Inspect interior bulkheads and stringers for damage,

cracks, and corrosion.					   	   ___	   ___	    ___

3.  Inspect coolant inlet for damage, loose or missing fasteners. 	   ___	   ___	    ___

4.  Inspect all antennas / mounts as installed for security,

cracks in structure, corrosion, sealing, wiring.  Check

cable ends / plugs for corrosion and proper connection.		   ___	   ___	    ___

5.  Inspect rudder cables for proper routing, condition,

tension, fraying, and chafing.  Check condition and

security of pulley brackets.  Rotate and inspect all pulleys

for wear.  Replace as necessary.					   ___	   ___	    ___

50	100	500	1000

6.  Inspect all manual rudder and elevator trim cables as

installed for proper routing, condition, tension, fraying,

and chafing.  Check condition  and security of pulley

brackets.  Rotate and inspect all pulleys for wear.

Replace as necessary.						   ___	   ___	    ___

7.  Inspect all wiring for electric rudder and elevator trim

as installed.							   ___	   ___	    ___

8.  Inspect trim drums and actuators for security and operation

through entire operating range, stop to stop.  Inspect trim

tabs and connecting rods, hardware, and safeties.  Check

for excessive play in trim rod ends, bolts, linkages,

hinges, and connections.  Lubricate.				   ___	   ___	    ___

9.   Inspect static vents and lines for condition, obstructions.		   ___	   ___	    ___

10. Check  all hoses, lines, and wiring for condition, proper

routing, chafing, security, and connections.			   ___	   ___	    ___

G.  Empennage						50	100	500	1000

1.   Inspect vertical fin and rudder for damage, distortion,

loose or missing rivets and fasteners.				   ___	   ___	    ___

2.  Check vertical fin rear spar for cracks, damage.  Inspect

spar bolts and attachments for condition and security.		   ___	   ___	    ___

3.   Inspect  rudder hinge brackets for cracks, wear,

proper hardware, safeties.					   ___	   ___	    ___

3.   Check rudder hinge bolts and bearings for wear.

Replace as required.						   ___	   ___	    ___

4.   Inspect rudder travel stops for condition and security.		   ___	   ___	    ___

5.   Check rudder travel stop to stop for 30 degrees each side

of center using forward cockpit rudder pedals to actuate.

Repeat using aft rudder pedals as installed.			   ___	   ___	    ___

6.   Inspect rudder trim tab and hinge for wear, proper

installation, operation, hardware, and safeties.			   ___	   ___	    ___

7.  Inspect rudder trim tab bolt and bearings for wear,

proper installation, operation, and safeties.			   ___	   ___	    ___

8.   Inspect horizontal stabilizer and elevator for damage,

distortion, loose or missing rivets and fasteners.			   ___	   ___	    ___

9.   Check stabilizer attachments and elevator hinge

brackets for cracks, wear, proper hardware, safeties.		   ___	   ___	    ___

10. Check elevator hinge bolts and bearings for wear.

Replace as required.						   ___	   ___	    ___

11. Inspect elevator travel stops for condition and security.		   ___	   ___	    ___

12. Inspect elevator bellcrank for condition, proper hardware,

and safeties.							   ___	   ___	    ___

13. Check elevator travel stop to stop for 30 degrees up and

20 degrees down, using forward cockpit control stick

to actuate.  Repeat with rear control stick.			   ___	   ___	    ___

14.  Inspect elevator control rods and rod ends and bearings for

condition, wear, proper travel, operation, and safeties.		   ___	   ___	    ___

15.  Inspect elevator trim tab and hinge for wear, proper

installation, operation, hardware, and safeties.			   ___	   ___	    ___

16.  Inspect rudder trim tab bolt and bearings for wear,

proper installation, operation, and safeties.			   ___	   ___	    ___

17.  Lubricate all empennage bolts, bearings, and hinges.		   ___	   ___	    ___

H.  Wing Group						50	100	500	1000

1.   Remove all inspection plates, panels, and fairings.			   ___	   ___	    ___

2.   Inspect all wiring, lines, and hoses in wings for condition,

chafing, proper routing, connections, and security.		   ___	   ___	    ___

3.   Inspect all surfaces for distortion, damage, loose or

missing rivets or fasteners.				___	   ___	   ___	    ___

4.   Inspect aileron hinges and attachments.				   ___	   ___	    ___

5.   Inspect condition and security of aileron balance weights.		   ___	   ___	    ___

6.   Inspect aileron control rods, rod ends, bearings, bellcranks,

and idlers for damage, routing, chafing, proper safeties.		   ___	   ___	    ___

7.   Inspect aileron hinges, attaching bolts and bearings for

cracks, wear, condition, and proper safeties.  Replace

bolts and bearings as necessary.					   ___	   ___	    ___.

8.   Inspect travel stops for condition and security.	  		   ___	   ___	    ___

9.   Check aileron travel stop to stop for 15 degrees up,

15 degrees down using forward control stick.  Repeat

with aft stick.	 (These values approximate – See Blueprint)	.	   ___	   ___	    ___

10. Inspect flaps for distortion, damage, loose or missing

fasteners and rivets.					___	   ___	   ___	    ___

11. Inspect flap bellcranks for damage, cracks, proper

fasteners, and security.					   	   ___	   ___	    ___

12. Inspect flap actuation torque tube for cracks, distortion,

and proper hardware.						   ___	   ___	    ___

13. Check condition and security of limit switches and stops.		   ___	   ___	    ___

14. Check flap settings and full flap stop at 40 degrees down.		   ___	   ___	    ___

15. Lubricate aileron and flap rod ends, hinges, bearings,

and actuating mechanisms.					   ___	   ___	    ___

16.  Inspect all wing mate structures for cracks, corrosion,

damage, loose and missing fasteners.				   ___	   ___	    ___

17.  Inspect fuel tank areas for leaks.				   ___	   ___	    ___

18.  Inspect all fuel lines from tanks to fuel selector valve

for leaks, security, and proper connection.			   ___	   ___	    ___

19.  Check fuel vents for damage, security, and obstructions.		   ___	   ___	    ___

20.  Inspect sending units for condition of wiring and leaks.		   ___	   ___	    ___

21.  Inspect condition of fuel tank caps, O Rings, and hardware.	  ___	   ___	    ___

22.  Check placards or decals for condition and proper marking

for fuel quantity, type, and octane.				   ___	   ___	    ___

23.  Inspect pitot tube for condition, security, and obstruction.		   ___	   ___	    ___

I.  Landing Gear Group.					50	100	500	1000

1.   Inspect oleo struts for proper extension (4” +/- 0.5 inches

at empty weight plus fuel, oil, and all fluids).		___	   ___	   ___	    ___

2.   Inspect tail wheel steering control and travel (6 degrees

right and left of center with stick back to engage locking

lever).  Check wear in lever and steering collar.			   ___	   ___	    ___

3.   Check wheels for alignment.					   ___	   ___	    ___

4.   Using jacks and proper tail stand, jack aircraft.			   ___	   ___	    ___

50	100	500	1000

5.   Inspect tires for cuts, uneven or excessive wear,

and slippage on wheels.					   ___	   ___	    ___

6.   Deflate tires.  Remove wheel / tire assemblies.  Remove,

clean, and inspect bearings for wear and corrosion.

Replace as necessary.  Repack with proper  lubricant.		   ___	   ___	    ___

7.   Inspect wheels for cracks, damage, corrosion, loose

and missing fasteners.  Inspect bearing races for wear

corrosion, and slippage in wheels.  Replace as necessary.		   ___	   ___	    ___

8.   Inspect brake linings and discs for damage, wear.

Replace as necessary.						   ___	   ___	    ___

9.   Inspect  brake cylinders and backing plates for damage,

condition, and leaks.						   ___	   ___	    ___

10. Inspect brake lines and hoses for leaks, chafing,

condition, security, and routing.  Check all connections

for security and leaks.						   ___	   ___	    ___

11. Inspect gear forks and axles for bends, cracks, and damage.

Reassemble and reinstall wheel / tire assemblies and safety.

Inflate Mains (45 psi) and tail wheel (50 psi) with nitrogen.

Reinstall brake assemblies, safety.				   ___	   ___	    ___

12. Inspect oleo struts for leaks, scuffs, and damage.	 		   ___	   ___	    ___

13. Inspect gear struts, attachments, torque links, and

trunion tubes for cracks, bent structures, and damage.

14. Check all bolts and bushings in torque links for wear,

proper safeties, and condition.					   ___	   ___	    ___

15. Inspect front trunion bearing housing for cracks, damage,

and evidence of shifting in stub spar.  Check mounting

bolts and nuts for damage, condition, and security.	___	   ___	   ___	    ___

16. Repeat above items for rear trunion bearing.  Inspect

closely for evidence of shifting in spar.			___	   ___	   ___	    ___

17. Inspect downlock plate, bolt, and jam nut for condition.	___	   ___	   ___	    ___

18. Inspect downlock plate mount for cracks, security of bolts.	___	   ___	   ___	    ___

19. Inspect downlock lever for cracks, stress, and damage.	___	   ___	   ___	    ___

20. Inspect idler lever for damage, cracks, and stress.		___	   ___	   ___	    ___

21. Inspect idler linkages for condition, mis-alignment; jam

nuts, rod ends, bolts for damage, proper safeties, and wear.___	   ___	   ___	    ___

22. Inspect idler trunions for shifting in spar and stub spar.

Check condition and security of mounting bolts.		___	   ___	   ___	    ___

23. Inspect downlock collar on trunion for cracks, damage,

evidence of stress.  Check security and condition of bolts

and nuts as installed.					___	   ___	   ___	    ___

23. Inspect linkages for condition, mis-alignment; jam nuts,

rod ends, bolts for damage, proper safeties, and wear.	___	   ___	   ___	    ___

24. Inspect all wiring routed into and through wheel wells

for proper routing, chafing, security, and connections.		   ___	   ___	    ___

25. Check all limit switches, warning horns as installed, and

gear indication lights for  proper adjustment, security,

and operation.			   			   ___	   ___	    ___

26. Check gear clear and retract gear, checking for proper

operation, gear doors for proper clearance and security,

and proper engagement of up locks.				   ___	   ___	    ___

27. Check gear clear and extend gear, checking for proper

operation, travel, and engagement in down locks.			   ___	   ___	    ___

28. Inspect actuating cylinders for leaks, damage, and security.		   ___	   ___	    ___

29. Inspect all hydraulic lines and hoses for leaks, condition,

security, routing,  and connections.				   ___	   ___	    ___

30. Check condition and operation of back up gear extension

system as installed.						   ___	   ___	    ___

31. Lubricate all grease fittings, rod ends, bearings, and

joint bolts.							   ___	   ___	    ___

50	100	500	1000

32. Check gear down and locked, remove aircraft from jacks.		   ___	   ___	    ___

J.  Operational Inspection.					50	100	500	1000

1.   Clean engine.  Secure cowling frames and

airframe for ground run of engine.		 	___	   ___	   ___	    ___

2.   Check fuel pump and fuel tank selector settings.	 	___	   ___	   ___	    ___

3.   Check all gages and instruments for proper

indications with master off, then with master on.   	___	   ___	   ___	    ___

4.   Start engine.  Check all gages and instruments

for proper operation and indications at idle.	 	___	   ___	   ___	    ___

5.   Check alternator performance and back up

electrical system as installed.				___	   ___	   ___	    ___

6.   After warm up, hold brakes and advance throttle

to 2500 engine RPM.  Check oil pressure, coolant

pressure for normal operating ranges.			___	   ___	   ___	    ___

7.   Check ignition systems independently as installed.	___	   ___	   ___	    ___

8.   Check propeller cycling and manifold pressure

indications.						___	   ___	   ___	    ___

9.   Retard throttle.  Check engine idle.  Shut down.		___	   ___	   ___	    ___

10. Check all gages and instruments for proper

indications.						___	   ___	   ___	    ___

K.  Reassembly						50	100	500	1000

1.   Reinstall all inspections plates, panels, cowling,

and fairings.						 ___	   ___	   ___	    ___

L.  General						50	100	500	1000

1.   Aircraft registration proper.				 ___	   ___	   ___	    ___

2.   Aircraft Flight Manual and / or Checklists as

supplied in order and good condition.			 ___	   ___	   ___	    ___

3.   Proper entries made in airframe, engine, and

propeller log books.  Aircraft returned to service.		 ___	   ___	   ___	    ___

Date Inspection Completed:  __________________________________________

End.